SMART INDIA HACKATHON 2026

TECHNICAL PROJECT REPORT

RESILIFORMA

A Deterministic Multi-Hazard Design Screening Extension for Autodesk Forma

Problem Statement ID: SIH26114
Problem Statement: Smart City Site Planning using Autodesk Forma Site Design
Category: Software / Miscellaneous
Theme: Urban & Rural Spatial Planning
Target Organization: Autodesk

Institution: Karunya Institute of Technology and Sciences
Location: Coimbatore, Tamil Nadu

Team: ResiliForma

1. ABSTRACT

Urban site planning requires designers to make decisions about buildings, roads, landscaping, environmental conditions and infrastructure before detailed engineering begins. During this early design stage, a small change in building orientation, landscape buffering or stormwater infrastructure can significantly affect the environmental performance of a proposed site.

ResiliForma is a supplementary decision-support extension designed to work alongside Autodesk Forma Site Design. It provides deterministic early-stage screening of three important environmental factors: road traffic noise, solar exposure and stormwater runoff.

The primary site design is created in Autodesk Forma. ResiliForma then provides an additional analysis layer that allows users to modify selected design parameters and observe their estimated effects. The system compares a conventional baseline proposal with a resilient proposal and produces a transparent multi-objective resilience score.

The prototype uses a React and TypeScript frontend, a Python FastAPI backend, Pydantic validation and deterministic mathematical calculation modules. The frontend provides an interactive dashboard containing a conceptual 2.5D SVG site visualization, parameter controls, analysis cards, proposal comparison and resilience scoring.

The acoustic screening model estimates attenuation from an acoustic barrier and vegetation buffer. The solar screening model estimates peak direct façade exposure reduction from building orientation and external louvers. The stormwater module uses the Rational Method to estimate peak runoff and evaluates the proposed bioswale and retention strategy.

For the demonstration scenario, the baseline proposal has a screening score of approximately 48/100, while the resilient proposal reaches approximately 87/100. The resilient scenario uses a -18° orientation adjustment, a 3.5 m acoustic berm, an 8 m vegetative buffer, 1.2 m louvers, a 644 m bioswale network and 7,800 m³ of retention capacity.

ResiliForma does not replace Autodesk Forma's native analyses or detailed engineering simulation. Instead, it acts as an early-stage screening and decision-support layer intended to help planners compare alternatives transparently before detailed design and engineering validation.

2. INTRODUCTION

Smart-city development requires the integration of multiple environmental, spatial and infrastructural considerations into the planning process. Conventional workflows often involve creating a conceptual site layout first and performing detailed environmental assessments later.

This creates a decision-making gap.

A planner may already have established the major building arrangement before discovering that:

- buildings are highly exposed to traffic noise;

- façades have excessive solar exposure;

- impervious surfaces produce substantial stormwater runoff;

- environmental mitigation requires expensive changes to the site layout.

ResiliForma addresses this early-stage decision-making problem.

The system is designed around the following principle:

Design in Autodesk Forma, screen environmental resilience with ResiliForma, compare alternatives, and continue detailed development using appropriate Autodesk workflows.

The system therefore does not attempt to reproduce Autodesk Forma. Autodesk Forma remains the primary site-design environment.

3. PROBLEM STATEMENT

3.1 Problem

The project addresses the Smart India Hackathon problem statement:

SIH26114 — Smart City Site Planning using Autodesk Forma Site Design.

The objective is to demonstrate a smart-city site-planning workflow using Autodesk Forma while considering environmental and planning factors during early-stage design.

The project focuses on three additional screening concerns:

- Road traffic noise

- Solar exposure

- Stormwater runoff

These factors are considered together rather than independently.

4. PROPOSED SOLUTION

ResiliForma is implemented as a supplementary analysis and decision-support layer around the Autodesk Forma workflow.

The overall process is:

Autodesk Forma

↓

Site Design

↓

ResiliForma

↓

Design Parameters

↓

Noise Screening

Solar Screening

Stormwater Screening

↓

Multi-Objective Resilience Score

↓

Proposal Comparison

↓

Design Decision

↓

Detailed Development / BIM Workflow

The system allows a planner to explore how design interventions influence environmental performance.

5. PROJECT OBJECTIVES

The major objectives of ResiliForma are:

- To use Autodesk Forma as the primary smart-city site-design platform.

- To provide an additional early-stage environmental screening layer.

- To screen road traffic noise using a deterministic acoustic heuristic.

- To screen solar exposure using orientation and louver parameters.

- To estimate stormwater runoff using the Rational Method.

- To allow interactive modification of design parameters.

- To compare baseline and resilient proposals.

- To provide a transparent composite resilience score.

- To visualize site elements through an original 2.5D interface.

- To maintain a clean separation between the Forma integration layer and the calculation engine.

- To provide a pathway for future integration with Autodesk Forma APIs and extensions.

- To support a transparent and reproducible decision-making workflow.

6. DESIGN PHILOSOPHY

A major design decision in ResiliForma is that the application must not attempt to clone Autodesk Forma.

Autodesk Forma is responsible for:

- site creation;

- site limits;

- building massing;

- transportation;

- landscaping;

- native Forma analyses;

- proposal development;

- Forma Board comparison.

ResiliForma is responsible for:

- additional screening;

- deterministic calculations;

- interactive parameters;

- multi-hazard comparison;

- resilience scoring;

- decision-support visualization.

Therefore:

Autodesk Forma = Primary Site Design Platform

ResiliForma = Supplementary Multi-Hazard Screening Layer

This separation prevents the application from becoming a replacement for the Autodesk platform.

7. DEMONSTRATION SITE

7.1 Site Location

The demonstration scenario represents:

Karunya Nagar Smart City Sector

Coimbatore South, Tamil Nadu, India

Approximate coordinates:

10.9366° N, 76.7441° E

7.2 Site Area

The demonstration site is defined as:

1.2 km²

Equivalent to:

1,200,000 m²

or:

120 hectares

The site is therefore larger than the 1 km² minimum used for the project demonstration.

7.3 Site Context

The conceptual site contains:

- building blocks;

- arterial road context;

- internal roads;

- pedestrian areas;

- vegetation buffers;

- acoustic buffering;

- bioswales;

- retention infrastructure.

The surrounding road context is represented by the NH-544 / Siruvani Road arterial corridor.

8. SITE DESIGN IN AUTODESK FORMA

The actual smart-city site is designed using Autodesk Forma Site Design.

The site-development workflow consists of:

- Defining the site boundary.

- Establishing the transportation context.

- Creating building massing.

- Adding landscaping.

- Developing alternative design proposals.

- Running available Forma site analyses.

- Comparing proposals using Forma Board.

ResiliForma does not recreate these functions.

Instead, the ResiliForma interface represents the environmental screening information associated with the design.

9. TWO SITE DESIGN PROPOSALS

The project uses two design alternatives.

9.1 Proposal 1 — Baseline

The baseline proposal represents a conventional site arrangement.

Parameters

- Orientation: 0°

- Acoustic barrier: 0 m

- Vegetation buffer: 0 m

- Louver depth: 0 m

- Runoff coefficient: 0.85

- Baseline noise: 78 dBA

- Baseline solar irradiance: 710 W/m²

Screening Result

Composite Resilience Score: 48/100

Grade: D

The baseline design represents a condition with relatively high exposure to the identified environmental hazards.

10. PROPOSAL 2 — RESILIENT DESIGN

The second proposal introduces environmental mitigation strategies.

Parameters

- Building orientation: -18°

- Acoustic berm: 3.5 m

- Vegetation buffer: 8 m

- External louver depth: 1.2 m

- Bioswale network: 644 m

- Retention capacity: 7,800 m³

Screening Result

Composite Resilience Score: 87/100

Grade: A

The resulting improvement is:

+39 resilience points

compared with the baseline screening scenario.

11. SYSTEM ARCHITECTURE

The system is divided into four major layers.

11.1 Layer 1 — Autodesk Forma

Autodesk Forma is the primary site-design platform.

It provides the conceptual site context and design proposals.

11.2 Layer 2 — ResiliForma Frontend

The frontend provides:

- dashboard;

- proposal selector;

- design controls;

- site visualization;

- noise panel;

- solar panel;

- stormwater panel;

- resilience score;

- comparison panel;

- methodology information;

- Forma integration information.

11.3 Layer 3 — FastAPI Backend

The backend receives normalized design parameters and performs deterministic calculations.

11.4 Layer 4 — Calculation Engine

Separate modules perform:

- acoustic screening;

- solar screening;

- stormwater screening;

- weighted scoring.

12. SYSTEM ARCHITECTURE DIAGRAM

The complete system can be represented as:

AUTODESK FORMA

│

│

Site Design

│

▼

Forma Integration Layer

│

▼

RESILIFORMA

│

React + TypeScript

│

▼

REST API

│

▼

FastAPI Backend

│

┌──────────────┼──────────────┐

▼              ▼              ▼

Noise          Solar       Stormwater

Engine         Engine        Engine

│              │              │

└──────────────┼──────────────┘

▼

Weighted Scoring

│

▼

Resilience Score

│

▼

Proposal Comparison

│

▼

Design Decision

│

▼

Detailed BIM Workflow

13. FRONTEND IMPLEMENTATION

13.1 Technologies

The frontend uses:

- React

- TypeScript

- Vite

- CSS

- SVG

The frontend is designed as an original interface rather than a recreation of Autodesk Forma.

14. RESILIFORMA DASHBOARD

The dashboard contains the major project information.

The main header displays:

RESILIFORMA

Deterministic Multi-Hazard Design Screening for Autodesk Forma

The dashboard identifies:

- SIH26114

- Autodesk Forma workflow

- demonstration site

- site area

- active proposal

- analysis results

15. SITE OVERVIEW

The site overview displays:

Karunya Nagar Smart City Sector

Coimbatore South, Tamil Nadu

1.2 km²

The conceptual site visualization contains:

- buildings;

- roads;

- vegetation;

- acoustic buffer;

- bioswales;

- retention area;

- pedestrian areas;

- site boundary.

16. 2.5D SVG SPATIAL VISUALIZATION

The frontend uses SVG to provide an interactive conceptual site representation.

The visualization is intentionally simplified.

It is not intended to reproduce the Autodesk Forma 3D modeling environment.

The visualization communicates the spatial relationship between:

- building blocks;

- roads;

- environmental buffers;

- vegetation;

- water-management infrastructure.

The SVG approach provides a lightweight visualization without requiring a complex 3D rendering engine.

17. INTERACTIVE DESIGN CONTROLS

The interface contains controls for the major intervention parameters.

Acoustic Barrier

Range:

0–5 m

Step:

0.1 m

Vegetation Depth

Range:

0–16 m

Step:

0.5 m

Louver Depth

Range:

0–2 m

Step:

0.1 m

Orientation

Range:

-30° to +30°

Step:

1°

Rainfall Intensity

Range:

40–100 mm/hr

Runoff Coefficient

Range:

0.40–0.95

Catchment Area

Range:

5–30 hectares

Bioswale Length

Range:

0–1000 m

Retention Capacity

Range:

0–10,000 m³

These controls allow the user to explore the effects of design changes.

18. FRONTEND TO BACKEND COMMUNICATION

The frontend communicates with the FastAPI backend through REST API requests.

The workflow is:

User changes parameter

↓

React state updates

↓

API request

↓

FastAPI

↓

Pydantic validation

↓

Calculation engine

↓

JSON result

↓

React dashboard

↓

Updated visualization

The API is designed around a normalized site-analysis payload.

19. BACKEND IMPLEMENTATION

The backend uses:

Python 3.11

FastAPI

Pydantic

Uvicorn

The calculation engine is divided into independent modules.

Example structure:

backend/

│

├── app/

│   ├── main.py

│   ├── schemas.py

│   │

│   └── calculations/

│       ├── noise.py

│       ├── solar.py

│       ├── stormwater.py

│       └── score.py

│

└── tests/

├── test_noise.py

├── test_solar.py

├── test_stormwater.py

└── test_api.py

This modular structure makes the calculation engine easier to test and extend.

20. API ENDPOINTS

The backend provides the following conceptual endpoints:

GET /

Provides basic service information.

GET /health

Checks backend availability.

POST /api/forma/analyze

Receives site/design parameters and returns screening results.

POST /api/forma/compare

Compares baseline and resilient proposal data.

GET /api/demo/site

Provides demonstration site information.

GET /api/demo/proposals

Provides predefined demonstration proposals.

21. PYDANTIC DATA VALIDATION

Pydantic is used to validate incoming data.

Parameters include:

- barrier height;

- vegetation depth;

- louver depth;

- orientation;

- baseline noise;

- baseline irradiance;

- runoff coefficient;

- rainfall intensity;

- catchment area;

- bioswale length;

- retention capacity.

This prevents invalid input values from being passed directly into the calculation engine.

22. ACOUSTIC SCREENING MODEL

The acoustic model is an early-stage deterministic screening heuristic informed by outdoor acoustic barrier and diffraction concepts.

The model is not a complete acoustic simulation.

The basic equation is:

Optimized Noise

=

Baseline Noise

− Barrier Attenuation

− Vegetation Attenuation

23. ACOUSTIC BARRIER CALCULATION

Barrier attenuation is calculated using:

A_barrier = min(18.0, barrier_height × 3.1)

For a 3.5 m barrier:

3.5 × 3.1

=

10.85 dB

24. VEGETATION ATTENUATION

Vegetation attenuation is calculated using:

A_vegetation

=

min(8.0, (vegetation_depth / 8.0) × 4.2)

For an 8 m vegetation buffer:

(8 / 8) × 4.2

=

4.2 dB

The total modeled attenuation is approximately:

15 dB

Therefore:

78 dBA

↓

approximately 63 dBA

25. ACOUSTIC RESULT DISPLAY

The frontend displays:

Baseline: 78 dBA

Resilient: approximately 63 dBA

Reduction: approximately 15 dB

The dashboard also displays the contributing interventions:

- 3.5 m acoustic berm;

- 8 m vegetation buffer.

26. ACOUSTIC LIMITATION

The acoustic model is a screening approximation.

It does not perform:

- full 3D acoustic ray tracing;

- boundary-element acoustic simulation;

- complex building-reflection modeling;

- certified acoustic compliance assessment.

The model is therefore presented as:

"Deterministic early-stage acoustic screening."

The previous proposal also explicitly identifies this limitation and states that the coefficients are prototype calibration parameters rather than direct coefficients from ISO 9613-2.

27. SOLAR SCREENING MODEL

The solar module estimates the reduction in peak direct façade solar exposure resulting from:

- Building orientation.

- External louvers.

The model is intentionally lightweight and deterministic.

28. ORIENTATION RELIEF

The orientation component is:

Orientation Relief

=

min(25.0, |orientation_offset| × 0.45 + 12.0)

The absolute orientation offset is used because the prototype evaluates the magnitude of orientation change.

29. LOUVER RELIEF

The louver component is:

Louver Relief

=

min(20.0, (louver_depth / 1.2) × 9.9)

30. TOTAL SOLAR REDUCTION

The combined result is:

Solar Reduction

=

min(45.0,

Orientation Relief + Louver Relief)

For the resilient demonstration:

Orientation = -18°

Louver depth = 1.2 m

The target modeled reduction is approximately:

32%

31. SOLAR RESULT DISPLAY

The dashboard communicates the result as:

Baseline

710 W/m²

↓

Resilient

approximately 480 W/m²

↓

approximately 32% modeled reduction

The displayed result is explicitly described as a screening estimate.

32. SOLAR LIMITATION

The solar model does not replace detailed solar or energy simulation.

It does not calculate:

- annual cooling energy;

- HVAC dynamics;

- whole-building energy consumption;

- detailed annual solar radiation;

- CFD-based thermal behavior.

It is intended for early-stage comparative screening.

33. STORMWATER SCREENING

The stormwater module uses the Rational Method for peak runoff estimation.

The main equation is:

Q_peak = 0.278 × C × I × A

where:

Q_peak = peak runoff in m³/s

C = runoff coefficient

I = rainfall intensity in mm/hr

A = catchment area in km²

34. STORMWATER DEMONSTRATION INPUTS

The demonstration scenario uses:

Catchment area = 21.04 ha

Rainfall intensity = 65 mm/hr

Runoff coefficient = 0.85

The area is converted:

21.04 ha / 100

=

0.2104 km²

The estimated unmitigated peak runoff is:

Q

=

0.278 × 0.85 × 65 × 0.2104

≈ 3.23 m³/s

35. STORMWATER INTERVENTION

The resilient proposal includes:

644 m contour bioswale network

and:

7,800 m³ retention capacity

The prototype produces an:

82% estimated runoff-management score

This percentage represents the prototype's management-performance score under defined storage and infiltration assumptions.

It does NOT mean an 82% reduction in peak discharge.

36. STORMWATER LIMITATION

The stormwater calculation is a lumped early-stage estimate.

It does not replace:

- detailed drainage-network modelling;

- 2D flood modelling;

- hydrodynamic simulation;

- professional drainage design.

The purpose is to allow early comparison of alternatives.

37. MULTI-OBJECTIVE RESILIENCE SCORE

The three main environmental results are converted into normalized scores.

Additional decision dimensions can include:

- acoustic performance;

- solar performance;

- stormwater performance;

- accessibility;

- land utilization.

The composite score is:

Composite Score

=

Σ(wᵢ × sᵢ)

where:

wᵢ = weight assigned to each criterion

sᵢ = normalized criterion score

The weights satisfy:

Σwᵢ = 1

The resulting score is represented on a:

0–100 scale

38. BASELINE SCORE

The baseline demonstration proposal produces:

48 / 100

Grade D

This represents the lower-performing alternative under the defined prototype scoring assumptions.

39. RESILIENT SCORE

The resilient proposal produces:

87 / 100

Grade A

The difference is:

87 − 48

=

39 points

Therefore:

+39 resilience points

The score is a project-specific decision-support index.

It is not an official government or Autodesk rating.

40. PROPOSAL COMPARISON

The dashboard provides a direct comparison between the two alternatives.

| Parameter | Baseline | Resilient |
| --- | --- | --- |
| Orientation | 0° | -18° |
| Acoustic barrier | 0 m | 3.5 m |
| Vegetation | 0 m | 8 m |
| Louvers | 0 m | 1.2 m |
| Baseline noise | 78 dBA | 78 dBA |
| Modeled noise | 78 dBA | ~63 dBA |
| Solar baseline | 710 W/m² | 710 W/m² |
| Modeled solar reduction | — | ~32% |
| Bioswale network | — | 644 m |
| Retention capacity | — | 7,800 m³ |
| Runoff management | Lower | ~82% |
| Composite score | 48/100 | 87/100 |
| Grade | D | A |

The comparison allows a planner to understand the effect of design interventions instead of examining isolated values.

41. INTERACTIVE USER WORKFLOW

The primary demonstration workflow is:

Step 1

Open ResiliForma.

Step 2

Load the Karunya Nagar Smart City demonstration site.

Step 3

Select:

Baseline Proposal

Step 4

Observe:

- 78 dBA noise;

- high solar exposure;

- lower stormwater performance;

- 48/100 resilience score.

Step 5

Select:

Resilient Proposal

Step 6

Observe:

- approximately 63 dBA;

- approximately 32% solar reduction;

- approximately 82% runoff-management score;

- 87/100 resilience score.

Step 7

Change the design sliders.

Step 8

Observe the updated deterministic screening results.

Step 9

Compare both proposals.

Step 10

Use the results as decision-support information for early-stage planning.

42. AUTODESK FORMA INTEGRATION

ResiliForma follows an extension-oriented architecture.

The intended workflow is:

Autodesk Forma

↓

Site Design

↓

Forma Extension

↓

ResiliForma

↓

Analysis Parameters

↓

FastAPI

↓

Screening Results

↓

ResiliForma Dashboard

The integration layer is deliberately separated from the calculation engine.

This allows the application to operate in demonstration mode without requiring Autodesk credentials.

43. FORMA ADAPTER ARCHITECTURE

The frontend uses an adapter concept for Forma integration.

Conceptually:

formaAdapter

│

├── Demo Adapter

│

└── Autodesk Forma Adapter

The demo adapter provides predefined site information.

The Autodesk adapter is reserved for the supported Forma extension/API workflow.

This means the calculation engine does not depend directly on Autodesk-specific implementation details.

44. DEMONSTRATION MODE

A fully functional demonstration mode is provided so that the prototype does not depend on live Autodesk credentials.

In demonstration mode:

- the site context is predefined;

- baseline proposal is predefined;

- resilient proposal is predefined;

- parameters can be modified;

- calculations are executed;

- results are displayed;

- proposal comparison works.

This ensures that the core ResiliForma prototype can be demonstrated independently.

45. IMPORTANT FORMA BOUNDARY

ResiliForma does not claim to replace Autodesk Forma's native site analyses.

Autodesk Forma remains responsible for the official site-design workflow and its supported analysis capabilities.

ResiliForma provides an additional screening layer.

The intended relationship is:

FORMA

Site Design + Native Analysis

+

RESILIFORMA

Additional Screening + Comparison

This distinction is central to the project.

46. FORMA BOARD WORKFLOW

The project uses two proposals:

Baseline

and

Resilient

These proposals can be presented and compared using Autodesk Forma Board.

ResiliForma complements this process by providing its additional screening metrics.

The workflow is:

Proposal 1

│

├──── Forma Board

│

Proposal 2

│

└──── Forma Board

│

▼

Proposal Comparison

│

+

ResiliForma Metrics

│

▼

Design Decision

47. REVIT WORKFLOW

After the early-stage site planning process, detailed development can continue through the Autodesk BIM workflow.

Conceptually:

Autodesk Forma

↓

Conceptual Site Design

↓

ResiliForma Screening

↓

Preferred Alternative

↓

Revit

↓

Detailed BIM Development

ResiliForma is therefore positioned before detailed engineering and BIM development rather than replacing those tools.

48. USER INTERFACE STRUCTURE

The main interface consists of:

Header

- ResiliForma logo/title

- SIH problem statement ID

- Forma workflow status

Site Overview

- Site name

- Location

- Site area

- proposal status

Spatial Canvas

- buildings

- roads

- vegetation

- acoustic buffer

- bioswales

- retention

Design Controls

- barrier height

- vegetation depth

- louver depth

- orientation

- rainfall

- runoff coefficient

- catchment

- bioswale

- retention

Analysis Cards

- Noise

- Solar

- Stormwater

Resilience Score

- overall score

- grade

- sub-scores

Proposal Comparison

- baseline

- resilient

- improvements

Methodology

- equations

- assumptions

- limitations

Forma Integration

- integration workflow

- demo mode

- extension architecture

49. SOFTWARE TECHNOLOGY STACK

| Layer | Technology | Purpose |
| --- | --- | --- |
| CAD/Site Design | Autodesk Forma | Primary site design |
| Frontend | React | UI component architecture |
| Language | TypeScript | Type-safe frontend development |
| Build Tool | Vite | Frontend development/build |
| Visualization | SVG | 2.5D conceptual site visualization |
| Backend | Python 3.11 | Calculation runtime |
| API | FastAPI | REST API |
| Validation | Pydantic | Request/response validation |
| Server | Uvicorn | ASGI server |
| Numerical Computing | NumPy | Numerical calculations |
| Spatial Computing | SciPy | Spatial calculations where required |
| Geometry | Shapely | Polygon/setback calculations where required |
| Testing | pytest | Automated testing |
| API Testing | HTTPX/TestClient | Backend API testing |
| BIM Workflow | Autodesk Revit | Detailed BIM development |

50. PROJECT DIRECTORY

The implemented prototype follows a modular project structure similar to:

ResiliForma/

│

├── frontend/

│   ├── src/

│   │   ├── components/

│   │   │   ├── Header.tsx

│   │   │   ├── SiteOverview.tsx

│   │   │   ├── ProposalSelector.tsx

│   │   │   ├── SpatialCanvas.tsx

│   │   │   ├── DesignControls.tsx

│   │   │   ├── NoisePanel.tsx

│   │   │   ├── SolarPanel.tsx

│   │   │   ├── StormwaterPanel.tsx

│   │   │   ├── ResilienceScore.tsx

│   │   │   ├── ComparisonPanel.tsx

│   │   │   ├── MethodologyPanel.tsx

│   │   │   └── FormaIntegrationPanel.tsx

│   │   │

│   │   ├── services/

│   │   │   └── api.ts

│   │   │

│   │   ├── integrations/

│   │   │   └── forma/

│   │   │       ├── formaAdapter.ts

│   │   │       └── demoFormaAdapter.ts

│   │   │

│   │   ├── types/

│   │   │   └── analysis.ts

│   │   │

│   │   ├── App.tsx

│   │   ├── main.tsx

│   │   └── index.css

│   │

│   └── package.json

│

├── backend/

│   ├── app/

│   │   ├── main.py

│   │   ├── schemas.py

│   │   └── calculations/

│   │       ├── noise.py

│   │       ├── solar.py

│   │       ├── stormwater.py

│   │       └── score.py

│   │

│   ├── tests/

│   │   ├── test_noise.py

│   │   ├── test_solar.py

│   │   ├── test_stormwater.py

│   │   └── test_api.py

│   │

│   └── requirements.txt

│

├── README.md

├── .gitignore

└── .env.example

51. API DATA FLOW

The analysis request contains parameters such as:

barrier_height

vegetation_depth

louver_depth

orientation_offset

baseline_noise_db

baseline_irradiance

runoff_coefficient

rainfall_intensity

catchment_area_ha

bioswale_length

retention_capacity

The backend validates these values and passes them to the appropriate calculation modules.

The response contains:

noise

solar

stormwater

score

This creates a clean separation between the frontend and the calculation engine.

52. ERROR HANDLING

The system includes validation and error-handling mechanisms.

The backend validates parameter ranges.

The frontend handles:

- loading states;

- API errors;

- unavailable backend conditions;

- invalid input;

- demonstration fallback.

The objective is to prevent a temporary backend issue from producing a completely unusable interface.

53. TESTING AND QUALITY ASSURANCE

Testing is performed on the deterministic calculation engine and API.

Test categories include:

Acoustic Tests

- barrier height bounds;

- vegetation bounds;

- attenuation calculation;

- final noise limits.

Solar Tests

- orientation limits;

- louver limits;

- reduction cap;

- irradiance calculation.

Stormwater Tests

- catchment conversion;

- Rational Method calculation;

- positive runoff;

- storage parameters.

API Tests

- request validation;

- endpoint response;

- proposal comparison;

- JSON schema integrity.

The earlier prototype documentation recorded automated tests covering mathematical bounds and payload schema validation.

54. PERFORMANCE

The calculation engine is designed to be lightweight because it uses deterministic screening equations instead of computationally expensive high-fidelity simulation.

The earlier prototype benchmark reported approximately:

124 ms

for the deterministic backend solver under local Linux-container conditions.

This measurement excludes:

- network latency;

- Autodesk Forma processing;

- frontend rendering.

Therefore, the value should be interpreted as a prototype solver benchmark rather than complete end-to-end application latency.

55. SECURITY

The application does not hardcode credentials.

Environment variables are used for configuration.

Example:

VITE_API_URL=http://localhost:8000

Autodesk credentials, if required for a future live integration, should be stored securely through environment/configuration mechanisms rather than source code.

56. LIMITATIONS

ResiliForma is an early-stage decision-support prototype.

It does not replace:

- detailed acoustic simulation;

- computational fluid dynamics;

- detailed hydrological modelling;

- structural engineering;

- whole-building energy simulation;

- professional engineering validation.

The acoustic model is a screening heuristic.

The solar model is a geometric screening heuristic.

The stormwater model is a Rational Method screening calculation.

The composite resilience score is a project-specific decision-support index.

These limitations are intentionally communicated to avoid presenting the prototype as a certified engineering system. The previous technical proposal similarly defined these model boundaries.

57. ADVANTAGES

The proposed system provides several advantages during early-stage planning.

57.1 Fast Screening

The deterministic models require relatively little computational effort.

57.2 Transparency

The user can see the equations and parameters contributing to the result.

57.3 Interactive Exploration

Design parameters can be changed through sliders.

57.4 Multi-Hazard View

Noise, solar and stormwater are considered together.

57.5 Proposal Comparison

Baseline and resilient alternatives can be directly compared.

57.6 Forma-Compatible Workflow

The system is designed to complement rather than replace Autodesk Forma.

57.7 Modular Architecture

The calculation modules are separated from the UI and Forma integration layer.

58. INNOVATION

The main innovation of ResiliForma is the combination of:

- early-stage environmental screening;

- multiple hazard categories;

- interactive design parameters;

- transparent deterministic calculations;

- proposal comparison;

- composite resilience scoring;

- Autodesk Forma-oriented workflow.

Instead of waiting until detailed engineering to identify environmental problems, the system encourages planners to consider resilience during conceptual site planning.

59. EXPECTED IMPACT

ResiliForma can help planners and architects:

- identify potential environmental issues earlier;

- compare alternative site arrangements;

- understand the effect of mitigation measures;

- make design decisions using transparent metrics;

- reduce unnecessary iteration;

- communicate environmental trade-offs to stakeholders.

The system is particularly suited to early-stage planning where rapid comparative feedback is more valuable than high-fidelity simulation.

60. DEMONSTRATION SCENARIO

The primary demonstration follows this sequence:

Stage 1 — Baseline

The planner opens the baseline proposal.

The dashboard displays:

Noise: 78 dBA

Solar exposure: high

Stormwater performance: lower

Resilience score: 48/100

Stage 2 — Resilient Design

The planner selects the resilient proposal.

The dashboard displays:

Noise: approximately 63 dBA

Solar reduction: approximately 32%

Runoff management: approximately 82%

Resilience score: 87/100

Stage 3 — Interactive Modification

The planner changes:

- barrier height;

- vegetation;

- louver depth;

- orientation.

The backend recalculates the screening results.

Stage 4 — Decision

The planner compares the alternatives and identifies the resilient proposal as the better-performing option under the defined screening model.

61. REAL-WORLD WORKFLOW

The intended practical workflow is:

1. Create site in Autodesk Forma

↓

2. Create baseline proposal

↓

3. Create resilient proposal

↓

4. Run appropriate Forma analyses

↓

5. Compare proposals in Forma Board

↓

6. Use ResiliForma for additional screening

↓

7. Review noise / solar / stormwater

↓

8. Compare resilience scores

↓

9. Select preferred concept

↓

10. Continue detailed BIM development

This workflow keeps the Autodesk platform at the center of the site-design process.

62. FUTURE ENHANCEMENTS

Future versions can extend the prototype with:

- More complete Autodesk Forma API integration.

- Automated extraction of supported site parameters.

- Automated result writeback where supported.

- Detailed acoustic simulation integration.

- EnergyPlus/OpenStudio validation.

- Detailed hydrological modelling.

- Advanced optimization algorithms.

- Automated alternative generation.

- More sophisticated spatial analysis.

- Extended Revit interoperability.

- Additional climate and accessibility indicators.

- Production deployment and security hardening.

These features are considered future development rather than mandatory components of the current prototype.

63. RISK ANALYSIS

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Screening model differs from detailed engineering simulation | Medium | Clearly label outputs as screening estimates |
| Forma API changes | High | Keep integration layer separated from calculation engine |
| Invalid user inputs | Medium | Pydantic validation and frontend range controls |
| User distrust of composite score | Medium | Display individual sub-scores and formulas |
| Complex site geometry | Medium | Use screening model only during early-stage planning |
| Lack of Autodesk credentials | High | Provide functional demonstration mode |
| Overclaiming engineering accuracy | High | Explicit scope and limitation statements |

64. TEAM CONTRIBUTIONS

Chris David Raj A

Team Lead and System Architect

Responsibilities:

- FastAPI architecture;

- system integration;

- Forma integration architecture;

- backend structure.

Jenisia Mary

Physics Screening Developer

Responsibilities:

- acoustic screening logic;

- solar orientation calculations;

- louver screening;

- integration of deterministic physics heuristics.

Harry Joseph

Candidate Ranking Developer

Responsibilities:

- weighted multi-objective scoring;

- proposal ranking;

- testing of scoring logic.

Chandy N Jomon

Civil / Hydrology Developer

Responsibilities:

- Rational Method runoff calculations;

- stormwater intervention modelling;

- spatial/hydrology-related logic.

Jeffrey

Frontend UI Engineer

Responsibilities:

- React interface;

- TypeScript components;

- SVG spatial visualization;

- interactive controls.

Kenrich Gladson

BIM / QA Lead

Responsibilities:

- testing;

- validation;

- BIM workflow;

- quality assurance.

65. CONCLUSION

ResiliForma provides a practical early-stage decision-support approach for smart-city site planning.

The system does not attempt to replace Autodesk Forma. Instead, it uses Autodesk Forma as the primary site-design environment and adds a supplementary deterministic screening layer.

The prototype focuses on three environmental factors:

Noise

Solar

Stormwater

Through a React-based interactive dashboard and Python FastAPI calculation engine, designers can modify key parameters and observe their estimated effects.

The demonstration compares a baseline proposal with a resilient alternative.

The baseline produces a screening score of approximately:

48/100

while the resilient proposal reaches:

87/100

The resulting:

+39 point improvement

illustrates how environmental interventions can be evaluated together during early-stage planning.

The most important contribution of ResiliForma is therefore not replacing detailed simulation, but enabling transparent and rapid exploration before detailed engineering begins.

The project demonstrates how a supplementary extension can connect conceptual site planning with multi-hazard decision support while maintaining a clear separation between deterministic screening, Autodesk Forma's native capabilities and professional engineering validation.

66. FINAL PROJECT STATEMENT

ResiliForma helps planners answer a simple but important question during early-stage urban design:

"Which site proposal is more resilient, and why?"

Autodesk Forma remains the primary platform for site design. ResiliForma adds a transparent screening layer for noise, solar exposure and stormwater, allowing designers to compare alternatives before moving into detailed engineering and BIM development.

67. REFERENCES

- Autodesk. Autodesk Forma Site Design and Autodesk Platform Services documentation.

- International Organization for Standardization. ISO 9613-2, Acoustics — Attenuation of Sound During Propagation Outdoors.

- Central Pollution Control Board. Noise Pollution (Regulation and Control) Rules, Government of India.

- Chow, V. T., Maidment, D. R., & Mays, L. W. Applied Hydrology. McGraw-Hill.

- NASA POWER. Climate and solar resource data.

- India Meteorological Department. Rainfall and hydrological climate information.

- ISRO Bhuvan. Indian geospatial and terrain information.

- OpenStreetMap. Open geographic transportation and road network data.

- Autodesk. Autodesk Forma Extension / Platform Services documentation.